Presray Corporation

Critical Containment

P.O. Box 200

Wassaic, NY 12592

Phone: (845) 373-9300

Fax: (845) 855 – 8034

www.presray.com

Email: technicalsupport@presray.com

Suggested Specifications For Model FB44

Side Hinged Flood Gate With Inflatable Gaskets

# Part 1 - General

- 1.01	Description

  - Work Included: Provide flood barrier(s) factory assembled with frame(s) and hardware in accordance with the contract documents.

- 1.02       Standards

  - Comply with the provisions of (as applicable).

1.   AWS Structural Welding Code.

- ASME Structural Welding Code Section IX.

- 1.03       Submittals

  - Manufacturers Data: Submit installation and maintenance instructions for flood barriers.

  - Shop Drawings: Submit shop drawings for flood barriers including dimensioned plans and elevations, sections, connections and anchorage, and parts list.

  - Calculations (Optional): Submit calculations, approved by a qualified engineer, to verify the barrier’s ability to withstand the design pressure loading.

- 1.04       Qualifications

  - Experience: The manufacturer of the flood barrier(s) shall present evidence attesting to at least 5 years of successful experience in the design and manufacture of both the flood barrier and flood barrier seal of the type specified.

# Part 2 - Products

- 2.01	Flood barrier shall be Model FB44 as manufactured by Presray Corporation.

- 2.02	Materials

  - Panel: Aluminum plate.

  - Conversion Frame: Low carbon steel (stainless steel optional).

  - Finish: Panel, bright aluminum finish.  Conversion frame, brush-off blast clean per SSPC-SP7, primed with one coat rust inhibitive, lead free, red primer.

  - Seals: Dual Presray type Pneuma-Seal® inflatable gaskets. Each seal shall have an automotive type air inflation stem and independent 0-60 PSI pressure gauge.

  - Hardware

Hinges: Presray six way adjustable hinges.  Low friction hinges with oil impregnated bronze bushing for radial and thrust loads.

Latches: Slide bolts to hold panel in closed position.

  - Optional:  For use where facility air is not available

Air Source: Control Panel with manifold and air tanks or hand pump (portable compressors also available for multiple flood barrier installations)

Nose Wheels on leading edge for extra wide openings.

Multi-Panel Systems with removable mullions between panels for openings too wide to be accommodated by single panel.

- 2.03	Design

  - Flood barrier(s) shall be designed with a minimum 2:1 factor of safety based on material yield strength, and shall provide an effective seal against the design flood level.

  - Panel and conversion frame shall have lower corners radiused to optimize sealing.

  - Conversion frame shall have mounting holes for expansion anchors and bolts (Options available include epoxy anchors for block walls, and studs for embedment in concrete).

- 2.04       Fabrication

  - Sealing surfaces shall be finished to 63 microinch to maximize sealing, uninterrupted by steps greater than .015, free from cracks, and with finish lay parallel to seal.

  - Frame to be straight within l/8” over entire length.

- 2.05       Inspection and Test

  - Proof test and leak test inflatable seals per Presray standard practice.

# Part 3 — Execution

- 3.01	Installation

  - Install flood barriers in accordance with manufacturer’s instructions and approved shop drawings.

# Part 4 — Warranty

- 4.01	1-year limited against defects and workmanship from date of shipment.